1. Core idea of the paper

This paper develops a probabilistic model for channel initiation: where hillslope flow becomes concentrated enough to incise a channel, rill, gully, or channel head. The authors argue that the old deterministic threshold

aSα≥C

is too rigid because the threshold Cis not constant in nature. It varies because sediment size, roughness, rainfall/runoff, vegetation, fire disturbance, and hydrologic conditions vary in space and time. So they treat Cas a random variable and compute a spatial probability of channel initiation, called PCI.

In plain terms:

Given a DEM-derived slope and contributing area at each grid cell, and uncertainty in sediment size, surface roughness, and runoff rate, what is the probability that this cell becomes channelized?

2. Overall goal

The goal is not to predict exact erosion volume. The goal is to predict the probability that a location transitions from hillslope flow to channelized flow.

The paper specifically tries to:

- Replace a single deterministic threshold Cwith a probability distribution.

- Derive Cphysically from sediment size, roughness, runoff, and shear-stress theory.

- Compare the modeled threshold distribution with observed channel-head thresholds from field data.

- Produce spatial maps of probability of channel initiation, PCI.

This is important because channelization strongly increases sediment transport. A cell may not be permanently channelized, but during an intense storm after wildfire, it may temporarily become a rill/gully/channel.

3. Main input and output

Input

The model uses two major input groups.

A. Spatial terrain inputs from DEM

Z(x,y)→S(x,y), a(x,y)

where:

| Variable | Meaning | Data type |
| --- | --- | --- |
| Z | DEM elevation | raster / matrix |
| S | local slope | raster / matrix |
| a | specific catchment area, upslope area per unit contour width | raster / matrix |
| aSα | terrain forcing index | raster / matrix |

The paper used a 30 m DEM to derive slope and contributing area.

B. Random hydrologic/surface inputs

| Variable | Meaning | Data type |
| --- | --- | --- |
| d50 | median grain size | random variable / field sample vector |
| nb | Manning roughness for bare soil | derived scalar random variable |
| na | additional roughness from vegetation, litter, obstructions | random variable |
| r | excess rainfall/runoff rate | random variable |
| T | soil transmissivity, used for saturation-excess runoff | scalar parameter |
| τc | critical shear stress | derived from d50 |
| C | channel initiation threshold | random variable |

Output

Main output:

PCI(x,y)=P(channel initiation at cell x,y)

So the final product is a probability raster/map, not a binary map.

Observed field data are used for validation:

Yj=ajSjα

at observed channel-head locations j=1,…,N.

4. Data structures in mathematical form

For a raster domain with Ngrid cells:

Z∈Rm×n
S=[Si]i=1N,a=[ai]i=1N
G=[aiSiα]i=1N

where Gis the topographic channel-initiation index.

For field channel heads:

Dobs={(xj,yj,aj,Sj,d50,j)}j=1Nh

where Nhis the number of observed channel heads.

For Monte Carlo simulation:

d50m, nam, rmm=1,…,M

Then:

Cm=f(d50m,nam,rm)

Finally:

PCIi≈1Mm=1MI(Cm≤aiSiα)

for infiltration-excess runoff.

5. Variables, distributions, and descriptive statistics

Trapper Creek

Trapper Creek was intensely burned in 1994, and extreme gullying occurred after a convective storm in 1995. The authors used field-mapped channel heads, local slopes, GPS locations, sediment samples, and DEM-derived contributing areas.

Important statistics:

| Variable | Distribution/statistics |
| --- | --- |
| d50 | lognormal |
| mean d50 | 2 mm |
| std. dev. d50 | 0.48 mm |
| r | uniform, 15–55 mm/h |
| na | uniform, 0.015–0.1 |
| Monte Carlo samples | 1500 |
| observed aSαmean | 26 m |
| observed aSαstd. dev. | 18.4 m |
| observed skewness | 1.13 |
| simulated Cmean | 26 m |
| simulated Cstd. dev. | 18.5 m |
| simulated skewness | 1.56 |

For a pointwise test, they used r=35mm/h and calibrated na=0.052. Median grain size explained about 40% of the observed variability in aSα, with R2=0.387and Nash-Sutcliffe NS=0.377.

Robert E. Lee Creek

REL Creek had two channel-initiation mechanisms:

| Type | Meaning |
| --- | --- |
| IEOF | infiltration-excess overland flow |
| SEOF | saturation-excess overland flow |

REL Creek sediment was coarser:

| Variable | Distribution/statistics |
| --- | --- |
| d50 | lognormal |
| mean d50 | 3.12 mm |
| std. dev. d50 | 1.05 mm |
| IEOF r | uniform, 20–30 mm/h |
| IEOF na | uniform, 0.043–0.1 |
| IEOF observed aSαmean | 70 m |
| IEOF observed std. dev. | 33 m |
| IEOF simulated Cmean | 73 m |
| IEOF simulated std. dev. | 39 m |

For SEOF, they used approximately:

T/r=600 m

with example values:

T=40 m2/d,r=0.066 m/d

or about:

r=2.75 mm/h

6. Process equations, step by step

Step 1: Total shear stress from overland flow

τt=ρwgyS

where:

| Symbol | Meaning |
| --- | --- |
| τt | total shear stress |
| ρw | water density |
| g | gravitational acceleration |
| y | flow depth |
| S | local slope |

This is the driving stress produced by water flowing downslope.

Step 2: Effective shear stress on bare soil

Not all shear stress acts on soil grains. Some is absorbed by vegetation, litter, rocks, and roughness elements. So:

τf=τtnbna+nb1.5

where:

| Symbol | Meaning |
| --- | --- |
| τf | effective shear stress acting on soil |
| nb | bare-soil Manning roughness |
| na | additional roughness |

Higher nameans vegetation/litter protects the surface, reducing effective shear.

Step 3: Bare-soil roughness from grain size

nb=kd50p

with:

k=0.0474,p=16

So coarser grains increase roughness.

Step 4: Flow depth from Manning equation

y=(nb+na)qoS1/23/5

where:

| Symbol | Meaning |
| --- | --- |
| qo | overland flow per unit contour width |

Step 5: Combine into effective shear-stress form

After substitution:

τf=b1qom1Sn1

where:

m1=0.6,n1=0.7

and:

b1=ρwgnb1.5(nb+na)-0.9

Step 6: Channel initiation criterion

A channel starts when:

τf≥τc

where τcis critical shear stress for sediment motion.

Step 7: Critical shear stress from Shields equation

τc=τ*g(ρs-ρw)d50

where:

| Symbol | Meaning |
| --- | --- |
| τ* | dimensionless critical Shields stress |
| ρs | sediment density |
| d50 | median grain size |

The authors used:

τ*=0.043

because simulated values varied only weakly over the field conditions.

7. Infiltration-excess runoff model

For infiltration-excess overland flow:

qo=ra

where ris excess rainfall/runoff rate.

Substituting into the shear-stress criterion gives:

aSα≥C

where:

α=n1m1=0.70.6=1.167

and:

C=τcρwgnb1.5(nb+na)-0.9rm11/m1

This Cis the channel-initiation threshold.

Key logic:

- Larger a: more contributing area → more water → easier channel initiation.

- Larger S: steeper slope → larger shear stress → easier initiation.

- Larger d50: coarser particles → larger τc→ harder initiation.

- Larger na: more vegetation/roughness → lower effective shear → harder initiation.

- Larger r: more runoff → easier initiation.

8. Saturation-excess runoff model

For saturation-excess runoff:

qo=ra-TS

or:

qo=r(a-as)

where:

as=STr

Here asis the specific catchment area needed for saturation.

Then the channel-initiation condition becomes:

(a-as)Sα≥C

or equivalently:

aSα≥C+B

where:

B=Sα+1Tr

So for SEOF, the threshold is more complex because the cell must first become saturated.

9. Probability of channel initiation

For infiltration-excess flow:

PCIie(x,y)=P(C≤aSα)

or:

PCIie=0aSαfC(C) dC

where fC(C)is the probability density function of C.

For saturation-excess flow:

C'=C+B
PCIse(x,y)=P(C'≤aSα)

or:

PCIse=0aSαfC'(C') dC'

The authors compute this mainly using Monte Carlo simulation and then show that the resulting Cdistribution is well approximated by a gamma distribution.

10. Gamma distribution for C

They found that the modeled and observed threshold distributions fit a gamma distribution well:

fC(C)=λkΓ(k)Ck-1e-λC

with:

μC=kλ
σC2=kλ2

So once mean and variance of observed or simulated Care known, the gamma parameters can be estimated.

11. Computational workflow

The paper’s computation can be written like this.

Step A: Prepare spatial terrain fields

From DEM:

Z→S, a

Then compute:

Gi=aiSiα

for every grid cell i.

Step B: Define probability distributions

Example for Trapper Creek:

d50∼Lognormal(μ,σ)
r∼U(15,55) mm/h
na∼U(0.015,0.1)

Step C: Monte Carlo sampling

For each Monte Carlo sample m:

d50m,rm,nam

Compute:

nbm=k(d50m)p
τcm=τ*g(ρs-ρw)d50m
Cm=τcmρwg(nbm)1.5(nbm+nam)-0.9(rm)m11/m1

Step D: Compute PCI per cell

For IEOF:

PCIi=1Mm=1MI(Cm≤Gi)

where:

Gi=aiSiα

For SEOF:

Bim=Siα+1Trm
PCIi=1Mm=1MI(Cm+Bim≤Gi)

Step E: Produce output

Final output:

PCI∈[0,1]m×n

a raster map where each cell has a probability between 0 and 1.

12. Spatial-temporal interpretation

Spatially, the model is raster-based. Each cell has a slope and specific catchment area. The random inputs d50, na, and rare treated as probability distributions rather than fully mapped spatial fields. The paper assumes spatially homogeneous distributions for these uncertain variables, meaning the model does not explicitly map spatial correlation of sediment size or roughness.

Temporally, the paper mainly tests a single-event case, especially the 1995 storm after the 1994 wildfire in Trapper Creek. But the authors argue that the same framework can be extended to longer-term management by using rainfall/runoff probability distributions over a design period.

So:

PCI=spatial probability under uncertain event/surface conditions

not a deterministic time-evolving erosion simulation.

13. What Figure 1 means conceptually

Figure 1 is basically a Bayesian-style information-flow diagram:

d50→nb, τc
nb, na, r, τc→C
S, a→aSα
C, aSα→PCI

So the model separates:

- terrain forcing: aSα

- resistance/threshold uncertainty: C

- final probability: P(C≤aSα)

14. Very important interpretation

This paper does not say every high-PCI cell is definitely a channel. It says:

High PCI=high likelihood of channelization under uncertain runoff/surface conditions

The observed channel heads tend to occur near the upper ends of high-PCI zones, which supports the theory. Figure 10 maps PCI for Trapper Creek and REL Creek, while Figures 11 and 12 show how PCI contours behave in slope-area space.

15. One-sentence summary for your work

This paper converts the classical deterministic channel-head threshold aSα≥Cinto a probabilistic raster model by treating Cas a random threshold derived from sediment size, roughness, and runoff uncertainty, producing a spatial probability map of channel initiation rather than a fixed channel/no-channel prediction.